PROJECT TECHNICAL REPORT

Shillong Land Use / Land Cover

Detection Pipeline

An Instance-Segmentation Approach to Multi-Class LULC Mapping using Satellite Imagery, GAN-Based Augmentation, and Semi-Supervised Mask R-CNN

Study Area:  Shillong, Meghalaya, India

Task:  Instance Segmentation — LULC Mapping

Model:  Mask R-CNN (ResNet-50 FPN)

Document Version 2.0

# Table of Contents

1. Introduction	3

1.1 Key Project Parameters	3

2. Dataset Overview	4

2.1 Image Sources	4

2.2 Valid-Pixel Statistics	4

3. Preprocessing Pipeline	5

3.1 Valid-Pixel Masking	5

3.2 Denoising	5

3.3 CLAHE Contrast Enhancement	5

4. Patch Extraction & Dataset Splits	6

5. GAN-Based Data Augmentation	7

6. Manual Annotation	8

6.1 Land Cover Classes (15 Categories)	8

6.2 Annotation Summary	8

7. Model Architecture & Training	9

8. Model Retraining – Cloud Class & Weighted Loss	9

8.1 WITH_CLOUD Dataset	9

8.2 Weighted Fast R-CNN Loss	10

8.3 Training Configuration	10

8.4 Training Loss Log	11

9. Validation & Detection Results	12

9.1 Sample Validation Patches with Detections	12

9.2 Sample Test Patches Used for Validation	12

9.3 Step 6 – Automatic Detection on Full Test Set	13

10. Output Artefacts	13

11. Pipeline Summary	13

# 1. Introduction

This report documents the end-to-end machine learning pipeline developed for Land Use / Land Cover (LULC) mapping of the Shillong region using high-resolution satellite imagery. The pipeline covers raw image ingestion, preprocessing, patch extraction, GAN-based data augmentation, manual annotation, and model training with Mask R-CNN for instance segmentation.

The dataset comprises 219 multi-source satellite images, including RESOURCESAT (RA3xx series), standard grid tiles (numeric IDs), and auxiliary image sets (AA-series and qwerty-series). Fifteen LULC categories were defined, covering agricultural, urban, forest, water, and transitional land-cover types.

## 1.1 Key Project Parameters

| Field | Detail |
| --- | --- |
| Study Area | Shillong, Meghalaya, India |
| Task | Instance Segmentation – LULC Mapping |
| Total Source Images | 219 |
| Land Cover Classes | 16 (15 LULC + Cloud_NoData) |
| Model Architecture | Mask R-CNN (ResNet-50 FPN) |
| Loss Function | Weighted FastRCNN Loss (class-balanced) |
| Augmentation | Classical + GAN-based synthetic patches |
| Annotation Tool | QGIS / VGG Image Annotator (VIA) |
| Annotation Format | COCO JSON |
| Final Training Loss | 0.4335 (epoch 10 / 10) |

# 2. Dataset Overview

## 2.1 Image Sources

| Source Prefix | Description | Approx. Count |
| --- | --- | --- |
| RA3xx | RESOURCESAT-2/2A (LISS-IV / PAN) | ~100 |
| AA-series | High-resolution auxiliary acquisition group A | ~50 |
| qwerty-series | Auxiliary acquisition group Q | ~40 |
| Numeric (2, 165, …) | Standard grid tiles | ~29 |

## 2.2 Valid-Pixel Statistics

Each image was analysed for valid (non-black) pixel coverage after masking. Images with less than 70% valid pixels were retained at the image level, but heavily-black 256×256 patches derived from them were excluded during patch extraction.

| Metric | Value |
| --- | --- |
| Total images processed | 219 |
| Successfully preprocessed | 219 |
| Mean valid pixel % | 78.2% |
| Minimum valid pixel % | 74.1% |
| Maximum valid pixel % | 84.4% |
| Median valid pixel % | 78.2% |

# 3. Preprocessing Pipeline

All raw satellite images passed through a three-stage preprocessing pipeline before being used for training or annotation.

## 3.1 Valid-Pixel Masking

A binary mask is created by thresholding the grayscale image at intensity 8. Pixels at or below this threshold are treated as sensor fill / border artefacts.

## 3.2 Denoising

OpenCV fastNlMeansDenoisingColored is applied (h = 5, hColor = 5, templateWindowSize = 7, searchWindowSize = 21) to suppress sensor noise without blurring edge detail.

## 3.3 CLAHE Contrast Enhancement

Contrast-Limited Adaptive Histogram Equalisation (CLAHE) is applied on the L-channel in LAB colour space (clipLimit = 2.0, tileGrid = 8×8). The enhancement is masked so that black-border pixels remain zero.

Figure 1 — Sample 256×256 enhanced patches sent for QGIS annotation.

# 4. Patch Extraction & Dataset Splits

Enhanced images were tiled into 256×256 patches using a non-overlapping stride (STRIDE = 256). Patches where fewer than 70% of pixels were valid were discarded to avoid training on mostly-black tiles.

| Metric | Value |
| --- | --- |
| Patch size | 256 × 256 px |
| Stride | 256 px (non-overlapping) |
| Min valid ratio | 70% |
| Total patches extracted | 11,479 |
| Train split (70%) | 8,035 |
| Validation split (15%) | 1,722 |
| Test split (15%) | 1,722 |

# 5. GAN-Based Data Augmentation

A Deep Convolutional GAN (DCGAN) was trained on the training-split patches to generate additional synthetic satellite-image patches for data augmentation. Two training rounds were conducted — an initial 10-epoch run and a refined 20-epoch run — with the following architectural improvements applied in round 2:

- Replaced ConvTranspose2d with Upsample + Conv2d to eliminate checkerboard artefacts.

- Applied label smoothing on real labels (0.9 instead of 1.0).

- Trained the generator twice per discriminator step to balance adversarial training.

- Reduced the discriminator learning rate (1e-4 vs. 2e-4 for the generator).

| Parameter | Value |
| --- | --- |
| Latent dimension | 100 |
| Image size (training) | 128 × 128 px |
| Batch size | 32 |
| Epochs (final run) | 20 |
| Training subset | 500 real patches |
| Optimizer | Adam (β₁ = 0.5, β₂ = 0.999) |
| Loss | Binary Cross-Entropy |
| Synthetic patches generated | 1,000 |
| GAN-augmented dataset total | 9,035 patches |

# 6. Manual Annotation

Manual polygon annotations were created using QGIS and the VGG Image Annotator (VIA) on a representative subset of 30 enhanced patches. Annotations were exported in VIA JSON format and converted to COCO instance-segmentation format. A semi-supervised pseudo-labelling pass subsequently extended the annotated set using the initially trained Mask R-CNN model.

## 6.1 Land Cover Classes (15 Categories)

| ID | Class Name | Manual Annotations |
| --- | --- | --- |
| 1 | Cropland | 6 |
| 2 | Fallow | 2 |
| 3 | Plantation | 0 |
| 4 | Sandy area | 2 |
| 5 | Scrub land | 9 |
| 6 | Mining | 3 |
| 7 | Rural | 0 |
| 8 | Urban | 3 |
| 9 | Deciduous | 22 |
| 10 | Evergreen | 4 |
| 11 | Forest plantation | 2 |
| 12 | Grass grazing | 3 |
| 13 | Inland wetland | 1 |
| 14 | Riverstream canals | 16 |
| 15 | Water bodies | 10 |

## 6.2 Annotation Summary

| Dataset Version | Images | Annotations |
| --- | --- | --- |
| Manual (QGIS + VIA) | 25 | 83 |
| Semi-supervised (pseudo-labels) | 633 | 1,605 |

# 7. Model Architecture & Training

Mask R-CNN with a ResNet-50 Feature Pyramid Network (FPN) backbone was used for instance segmentation. The model was initialised with ImageNet pre-trained weights and fine-tuned on the Shillong LULC dataset. A semi-supervised learning loop was used to iteratively expand the labelled set using high-confidence pseudo-labels generated by the model itself.

| Component | Configuration |
| --- | --- |
| Architecture | Mask R-CNN |
| Backbone | ResNet-50 + FPN |
| Pre-training | COCO (ImageNet weights) |
| Number of classes | 15 LULC + background |
| Input resolution | 256 × 256 px patches |
| Semi-supervised iterations | Iterative pseudo-label expansion |
| Saved model | outputs/maskrcnn_shillong_lulc.pth |

# 8. Model Retraining – Cloud Class & Weighted Loss

After the initial training run, cloud-covered patches were identified and added as a dedicated 16th class (Cloud_NoData) to improve robustness on cloudy imagery. The model was rebuilt from scratch with 17 output nodes (16 classes + background) and retrained on the extended WITH_CLOUD dataset using a custom weighted-loss function to address class imbalance.

## 8.1 WITH_CLOUD Dataset

| Metric | Value |
| --- | --- |
| Total images | 642 |
| Annotations | 1,614 |
| Train split (85%) | 546 images |
| Validation split (15%) | 96 images |
| Extra class added | Cloud_NoData (class 16) |
| Total classes | 16 LULC + background = 17 |

## 8.2 Weighted Fast R-CNN Loss

To counter class imbalance, a custom weighted_fastrcnn_loss function replaced the default Faster R-CNN box/classification head loss. Class weights were computed inversely proportional to annotation frequency and clipped at 15.0 to prevent extreme gradient scaling.

### Per-Class Weights (16 Classes)

| Class | Weight |
| --- | --- |
| Cropland | 15.000 |
| Fallow | 15.000 |
| Plantation | 1.000 |
| Sandy_area | 15.000 |
| Scrub_land | 0.842 |
| Mining | 15.000 |
| Rural | 1.000 |
| Urban | 15.000 |
| Deciduous | 0.300 |
| Evergreen | 15.000 |
| Forest_plantation | 15.000 |
| Grass_grazing | 15.000 |
| Inland_wetland | 15.000 |
| Riverstream_canals | 0.454 |
| Water_bodies | 0.300 |
| Cloud_NoData | 12.810 |

The loss combines cross-entropy classification loss (with class weights applied) and Smooth-L1 box-regression loss (beta = 1/9, reduction = 'sum', normalised by total label count).

## 8.3 Training Configuration

| Parameter | Value |
| --- | --- |
| Architecture | Mask R-CNN (rebuilt fresh) |
| Backbone | ResNet-50 FPN (DEFAULT weights) |
| Output classes | 17 (16 LULC + background) |
| Epochs | 10 |
| Device | CUDA (GPU) |
| Loss function | Custom weighted FastRCNN loss |
| Box regression loss | Smooth-L1 (beta = 1/9) |
| Optimiser | SGD (torchvision default) |
| Saved model | maskrcnn_FINAL_WITH_CLOUD.pth |

## 8.4 Training Loss Log

The model converged steadily over 10 epochs. Total training time was 1,259 seconds (~21 minutes) on GPU.

| Epoch | Loss | Elapsed (s) |
| --- | --- | --- |
| 1 / 10 | 0.9853 | 115 |
| 2 / 10 | 0.8826 | 236 |
| 3 / 10 | 0.8240 | 361 |
| 4 / 10 | 0.7298 | 489 |
| 5 / 10 | 0.6632 | 617 |
| 6 / 10 | 0.6197 | 745 |
| 7 / 10 | 0.5573 | 873 |
| 8 / 10 | 0.5198 | 1,002 |
| 9 / 10 | 0.4865 | 1,131 |
| 10 / 10 | 0.4335 | 1,259 |

Figure 2 — Training loss curve over 10 epochs (GPU, WITH_CLOUD dataset).

# 9. Validation & Detection Results

After training, the model was evaluated on the validation set (96 images). The final model with the Cloud_NoData class successfully detected and localised LULC features including river/stream networks, forest types, scrub land, water bodies, and cloud-covered areas. Sample detections below demonstrate bounding-box predictions with confidence scores across diverse land-cover types.

## 9.1 Sample Validation Patches with Detections

The validation check (final_validation_with_cloud.png, generated in Colab) showed the model correctly identifying multi-class scenes. Example predictions include:

- Riverstream_canals + Scrub_land on high-resolution river-channel imagery.

- Deciduous + Cloud_NoData on partially cloud-covered forest patches.

- Evergreen + Deciduous + Cloud_NoData on mixed forest / cloud scenes.

- No detections (correctly blank) on fully cloud-obscured patches.

## 9.2 Sample Test Patches Used for Validation

The following patches are representative samples from the test set that were passed through the trained model during final evaluation.

Figure 3 — Representative test patches from the final evaluation batch.

## 9.3 Step 6 – Automatic Detection on Full Test Set

The final pipeline step (Step 6) ran the semi-supervised Mask R-CNN over the complete test set, performing automatic detection, localisation, and classification of LULC instances. Detected classes included Water_bodies, Scrub_land, Deciduous, Evergreen, Riverstream_canals, Cloud_NoData, and others. Some patches yielded no detections (correct behaviour for fully cloud-covered or featureless tiles). The final demonstration figure was saved as STEP6_final_demonstration.png in Colab.

| Observation | Description |
| --- | --- |
| Multi-class patches | A single 256×256 patch often contains 3–5 LULC classes simultaneously |
| Cloud detection | Cloud_NoData correctly flagged on white/hazy regions (confidence ≥ 0.60) |
| Forest types | Deciduous and Evergreen separated with moderate confidence (0.57–0.79) |
| River networks | Riverstream_canals detected with high recall on linear water features |
| Scrub land | Scrub_land detected on dry/sparse vegetation areas |
| No detection | Fully opaque cloud patches correctly returned no LULC predictions |

# 10. Output Artefacts

| File / Folder | Description |
| --- | --- |
| outputs/enhanced/ | CLAHE-enhanced images (219 files) |
| outputs/masks/ | Binary valid-pixel masks (.png + .npy) |
| outputs/patches/ | 11,479 tiled 256×256 patches |
| outputs/dataset/ | Train / val / test splits |
| outputs/gan_generated/ | 1,000 GAN synthetic patches |
| outputs/dataset_v3_gan_augmented/ | 9,035 patches (real + GAN) |
| outputs/coco_dataset/annotations_coco.json | Manual COCO annotations (83 instances) |
| outputs/coco_dataset/annotations_coco_SEMISUPERVISED.json | Semi-supervised COCO annotations (1,605 instances) |
| outputs/coco_dataset/annotations_coco_WITH_CLOUD.json | WITH_CLOUD dataset annotations (1,614 instances, 16 classes) |
| outputs/gan_generator_final.pth | Saved GAN generator weights |
| outputs/maskrcnn_shillong_lulc.pth | Saved Mask R-CNN weights (initial) |
| maskrcnn_FINAL_WITH_CLOUD.pth | Final Mask R-CNN weights (16 classes + cloud) |
| outputs/preprocessing_metadata.json | Per-image preprocessing statistics |

# 11. Pipeline Summary

| Stage | Input | Output | Key Numbers |
| --- | --- | --- | --- |
| 1. Ingestion | Raw satellite images | Validated file list | 219 images |
| 2. Preprocessing | Raw images | Enhanced + masked images | 219 successful, ~78% valid px |
| 3. Patch extraction | Enhanced images | 256×256 patches | 11,479 patches |
| 4. Dataset split | All patches | Train / val / test | 8,035 / 1,722 / 1,722 |
| 5. GAN training | Train patches | Trained GAN model | 20 epochs, 500-patch subset |
| 6. GAN generation | Trained GAN | Synthetic patches | 1,000 generated |
| 7. Annotation | 30 patches | COCO polygons | 83 instances, 15 classes |
| 8. Semi-supervised | All patches + model | Pseudo-labels | 1,605 instances, 633 images |
| 9. Cloud class added | COCO + cloud patches | WITH_CLOUD dataset | 642 images, 1,614 annotations |
| 10. Retraining | WITH_CLOUD dataset | Final Mask R-CNN | 10 epochs, loss 0.985 → 0.433 |
| 11. Evaluation | Test set | Detections + masks | 16 LULC classes, GPU inference |

The complete pipeline successfully processed 219 satellite images of Shillong, generating over 11,000 training patches and a semi-supervised annotation set of 1,605 polygon instances. A Cloud_NoData class was added as the 16th category, expanding the WITH_CLOUD dataset to 642 images and 1,614 annotations. The final Mask R-CNN (ResNet-50 FPN) was retrained with a weighted-loss function over 10 epochs, reducing training loss from 0.985 to 0.433 and achieving reliable multi-class LULC detection on the full test set.